#### eneo GL-Manager

Installation & Operation Guide

Version1.2

## The eneo GL-Manager video management software

This section provides instructions for installing and using the eneo GL-Manager which is included with the device. The programs can be operated by a selected PC equipped with the following requirements.

System Requirements

- Intel Pentium 1.5 GHz processor or above (2.8 GHz or higher recommended).

- 128-MB RAM at least (256-MB or higher recommended).

- Windows 2000, XP or above.

- 4 MB VGA card capable of 24-bit true color display.

- 100 MB free hard-disk space for software installation.

- 10/100 Base-TX network for LAN operation.

- The Microsoft Internet Explorer 6.

In the multi-channel recording mode, the HD data transfer rates must be 66MB or above (100MB or higher recommended).

### 1.1  Introduction to the eneo GL-Manager

The eneo GL-Manager allows you to access many units of the device from a remote desktop or a laptop in a TCP/IP networking environment. It can perform the following functions.

- Views live images in a smooth sequential flow from a camera.

- Stores, searches, and reviews recorded video from a PC or an SD card quickly.

- Changes manual record, event record, and schedule record settings easily.

Before you are ready to view images from a desktop, you need to have your device networked by obtaining a 10/100 base-TX Ethernet data cable (Standard RJ-45) to connect the camera to your LAN/WAN. Now enter the main menu to set the IP address.

### 1.2  Install the eneo GL-Manager in your PC

Install the eneo GL-Manager from the supported CD-R.

1. Exit all applications currently running in the selected PC.

2. Insert the supported CD in the CD-ROM drive. The program will execute the installation automatically. Follow the on-screen instructions to proceed with the rest of the installation procedure as they appear.

NOTE: The program will update the .NET Framework to version 2.0 automatically. Please click “Next” to start and finish the setup process.

3. Select a language of the installing process from the drop-down list in the “Installer Language” window.

4. After the installation is complete, pop up the START menu from your computer, and point to Programs/ eneo GL-Manager/ eneo GL-Manager to open up the program selection page. Click the eneo GL-Manager tag to start the eneo GL-Manager program.

Install the eneo GL-Manager for the ZIP file.

1. Save the ZIP file to your PC and extract the file to a designated directory.

2. Open the extracted folder. The folder contains 1 file.

3. Click the setup icon to execute the installation and then follow the on-screen instructions to proceed with the rest of the installation procedure.

4. After the installation is complete, click the START menu from your computer, and point to Programs / eneo GL-Manager to start the eneo GL-Manager program.

NOTE: Please make sure the TCP/IP communication software has been properly set and configured in your computer.

### 1.3  Login the software

Once the eneo GL-Manager is executed, a Login prompter will appear. You must enter the default user name: admin, and the password: 9999 in the respective spaces (and confirm the password again the first time you log in). Click the “OK” button and enter the console page of the eneo GL-Manager: both the user name and password must be entered correctly. Click the “Cancel” button and exit the login of the eneo GL-Manager.

View the video from a remote PC

Follow the instructions below to use the eneo GL-Manager to browse a camera video from a remote location. Upon entering the eneo GL-Manager, a connection box will appear as follows.

- Press the  (Add Device) button on the Device Management page.

- Add a channel from the Add Device page.

- (1) Click the Auto Search button at the bottom of the "Add Device" page to discover the connection of the device in the LAN. Instantly the "Search Device" page will appear. Click the device of your choice and click "Add" at the bottom of the page to access the "Add Device " page again.

(2) This page provides the IP Address and the HTTP Port. After typing the Device Tile, Username, Password and selecting the share device function or the permissions function, please click the "Add " button to establish the connection between the device and the computer.

- Click the device title to begin viewing images with this camera from the device list.

NOTE: To add more connections or units, please repeat the above instructions.

NOTE: Please click the Test Connection icon to test the connecting situation and preview the video of the camera.

### 1.4  Operation

1.4.1 Device Management

Once the connection has been established, click the  button to enter the Device Management window ( see the sample screen below.) On the left side of the window is the connected device that has been arranged when you established the connection.

|  |  | Add device: Add a new device. For more details please refer to section 1.3. |
| --- | --- | --- |
|  |  | Modify device: Modify the added device. Please select a device, then click this button to edit the device. |
|  |  | Delete device: Delete the added device. |
|  |  | Device Setting: For the further setting of each channel, please select the device title on the left side and click the setting button to enter the setting pages of the device. |
|  |  | Schedule Setting: For the further setting of each channel, please select the device title on the left side and click the Schedule button to enter the schedule setting pages of the device. |
|  | List of devices: The users can click the title of the device and use the icons above the list to change the device settings. |  |
|  | Display area: Click the title of the device, and you will see the live image in this area. |  |

NOTE:

The Device Setting page:

For the further settings of each channel, please select the device title on the left side and click the setting button to enter the setting pages of the device.

Click the “Setting” button to bring up the device-setting pages. You can interchange between four buttons: the “Image”, “Network”, “System” and “Application”.

The method of operation is the same as the Microsoft Internet Explorer setting pages.

NOTE:

The Schedule Setting page:

Click the  icon to enter the Schedule setting page.

|  | The setting area: |  |  |
| --- | --- | --- | --- |
|  |  |  | Sets the recording day from the drop-down list. Note: After the setting of the starting and ending time is done, select “Daily” to add the same time schedule from Sunday to Saturday. |
|  |  |  | Sets the starting time of recording from the drop-down list. |
|  |  |  | Sets the ending time of recording from the drop-down list. |
|  |  |  | Clicks the icon to enable the just added schedule. |
|  |  |  | Clicks the icon to disable the selected schedule. |
|  | The working area: |  |  |
|  | (1) | Click the working area directly to add a new schedule as you wish. |  |
|  | (2) | The functioning schedule bar is colored orange and the non-functioning schedule bar(s) is colored pink. You can just click the right button of the mouse on the functioning schedule bar ( in orange color) to cancel it. |  |
|  | (3) | To enlarge the display size of the working area, please click the left button of the mouse on the numeral bar ( …) ; on the other hand, please click the right button to narrow it. Note: The largest column length is 4 times the default size of the working area.. |  |
|  | The schedule bar: |  |  |
|  | (1) | Click and drag the left and right edges of the schedule bar to adjust the starting and ending time of the schedule. Note: For the exact schedule period setting, please set the day and time in the setting area directly. |  |
|  | (2) | Click and drag the middle of the schedule bar to move it. The moved bar is in the same length but the starting and ending times are changed. |  |
|  | Save: Apply the changes. |  |  |
|  | Undo: Undo the latest changes. |  |  |
|  | Clear All: Clear all the schedule markers in the current editing area. |  |  |

1.4.2 Live View

Press the  button to enter the Live View page.

|  |  | Add Live View Group: Press to add a new group of the live view. To change the group name, please left click twice on the title of the group, and then enter the new title. |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Group Sequence: This button enables you to see the sequential display of the group. Please select the group, and then click this button to start the display. |  |  |  |
|  |  | Delete Live View Group: 1.) Select the Live View Group you want to delete from the list. 2.) Press “Yes”. |  |  |  |
|  |  | Add Sub Group: 1.) Click the button to open the Add Sub Group window. 2.) Select the Square Division from the drop-down list: “Division 1” (single window), “Division 4” ( 4-windows), “Division 9” (9-window) and “Division 16” (16-window). 3.), Now press the “Add” button. |  |  |  |
|  |  | Modify Sub Group: 1.) Click the button to open the Modify Sub Group window. 2.) Select the Square Division from the drop-down list: “Division 1” (single channel), “Division 4” ( 4-windows), “Division 9” (9-window) and “Division 16” (16-window), 3.) Check mark the “Cover Original Name” or don’t, 4.) Press “Modify” button. |  |  |  |
|  |  | Delete Sub Group: 1.) Select the division mode you want to delete from the list. 2.) Now press “Yes”. |  |  |  |
|  | List of divisions and its groups: Click twice on the title of the list to change the groups’ names or the divisions’ names. |  |  |  |  |
|  | Display area: Click the title of the division, and you will see the live image in this area. |  |  |  |  |
|  | Audio function: |  |  | The audio function is not available. |  |
|  |  |  |  | The audio function is available. Press to activate the function and the icon will turn purple “ ”. Press again to deactivate the function. |  |
|  | Recording function: |  |  | The recording function is not available. |  |
|  |  |  |  | The recording function is available. Press to activate the function and the icon will turn red “ ”. Press again to deactivate the function. |  |
|  | PTZ Control Panel: |  |  | Press the button to expand the PTZ Control Panel; Press the button to minimize the PTZ Control Panel |  |
|  |  |  |  | , , …: The direction keys |  |
|  |  |  |  | Speed: Here you have a drop down list of speed dome rates from 1 to 63. 1 is the lowest rate and 63 the highest. You can set any one of these to control your speed dome rate. |  |
|  |  |  |  | (Preset): Click this button to save the preset position. |  |
|  |  |  |  | (Preset Goto): Click this button to automatically view the preset position number you have selected. |  |
|  |  |  |  | (Preset Clear): Choose any one preset position and click this button to delete your choice. |  |
|  |  |  |  | (Preset Position number): There's a drop down list here from which you can choose the preset position you want. |  |
|  | (Auto Scan): Click this button to view preset positions 1 to 8 automatically. |  |  |  |  |
|  | (Stop Auto Scan): Click this button to stop auto scan. |  |  |  |  |
|  | , and : The Tour function keys. |  |  |  | and : The Zoom function keys. |
|  | and : The Aperture function keys. |  |  |  | and : The Focus function keys. |
|  | : Flip: Click this button to turn the camera180 degrees horizontally. |  |  |  |  |
|  | : Zero pan: This button enables you to return the camera to the original direction it was facing. |  |  |  |  |
|  | : OSD: Click this button to activate the OSD in the dome device menu -- the OSD enables you to select or change your remote settings. |  |  |  |  |

NOTE: Add a camera video onto the Live View area: (1) Click the title of the camera that was connected. (2) Right click on the live view area, then click left to select a camera from the list. The video from the new camera will be displayed. (3) In the Live View area, if you want to change/delete the videos of cameras or use the snapshot/ full screen function, you can right click again on the live view area, then click left to select.

NOTE: When there is a detection of external devices such as a sensor, it will display a red color bar on the title displaying area and make an alarm sound to warn the user.

NOTE: How to set the preset position: Choose a preset position number, and then use the "Tilt" and "Pan" direction keys to find the camera angle you want. Now click the "Preset" button to save your position. You can similarly set other preset positions -- please note -- by first changing the position number. Now click the "Auto Scan" button to view preset positions 1 to 8 automatically.

1.4.3 eneo GL-Manager Playback

Press the  button to enter the eneo GL-Manager Playback page.

|  |  | Click to choose the recording list by channel. |  |  |
| --- | --- | --- | --- | --- |
|  |  | Click to choose the recording list by time search. |  |  |
|  |  | Enter the MONTH /DAY /YEAR /HOUR /MINUTE you wish to search and click the button to proceed. |  |  |
|  | NOTE: The recorded video list box will show the recorded video which are stored in the HDD of the connected devices by channel or by time. |  |  |  |
|  |  |  |  | : Click to use the playback-controlling panel. |
|  |  |  |  | : Click to display the volume control panel. |
|  |  |  |  | / / : Pause/ play / stop. "Play": Click to stop playing back the recorded video. "Pause": Click to freeze the image. "Stop": Click to stop playing back the recorded video. |
|  |  |  |  | / : Slow motion rewind/ forward. Slow reverse and forward (1x, 2x, 4x, 8x, 16x). |
|  |  |  |  | / : Speed up rewind/ forward. Fast reverse and forward (1x, 2x, 4x, 8x, 16x). |
|  |  |  |  | / : Goto previous/ next record. Click to select the previous/ next recorded video. |
|  |  |  |  | / : Step back/ forward. Click to play a recorded video step by step in the reverse/ forward direction. |
|  |  |  |  | : Click to set the export function. The function fits the starting and ending points of an image display, when the user wants to transfer a file. |
|  |  |  |  | Start/ End time: Set the start and end time that you want to export (backup) the recorded files to the AVI files. Note: You also can click to select the recorded file on the recording list above. |
|  |  |  |  | : Click to add the whole selected video to the arranged exporting list. |
|  |  |  |  | : Click to add the selected part of the recorded video (with time setting) to the arranged exporting list. |
|  |  |  |  | : Click to reset the Start/End time. |
|  |  |  |  | : Click to start to run the export function. |
|  |  |  |  | : Click to run the export function of the selected work on the list. |
|  |  |  |  | : Click to delete the arranged work. |
|  | NOTE: To change the folder of saving the exported AVI files please go to the Config setting page (Config\ Custom\ Path Settings\ Folder for Export AVI Clips\). |  |  |  |
|  | The display area. |  |  |  |
|  |  |  | Provides the image capturing function. Click the right key of the mouse on display area to show this icon, then click left to capture a jpeg file of the video. |  |
|  | Scroll bar for video searching: Drag the square on the bar to search the video. You can click or pull the indicator on the scroll bar to the point you want to see. |  |  |  |

1.4.4 DVR Remote Playback (for DVR devices only)

Press the  button to enter the DVR Remote Playback setting page. This setting page is for connecting with the DVR devices

|  | Device: | Click to choose the recording list by device. |
| --- | --- | --- |
|  |  | Enter the MONTH /DAY /YEAR /HOUR /MINUTE you wish to search and click the button to proceed. |
|  | Export: | Click to add the whole selected video to the arranged exporting list. |

NOTE:

To change the folder of saving the exported AVI files please go to the Config setting page (Config\ Custom\ Path Settings\ Folder for Export AVI Clips\).

|  | First: | Back to the first recording list page. |
| --- | --- | --- |
|  | Prev: | Back to the previous recording list page. |
|  | Next: | Goto the next recording list page. |

NOTE:

The recorded video list box will show the recorded video which are stored in the HDD of the connected devices by time.

|  |  |  | Control: Click to use the playback-controlling panel. |
| --- | --- | --- | --- |
|  |  |  | : Click to display the volume control panel |
|  |  |  | / / : Pause/ play / stop. "Play": Click to stop playing back the recorded video. "Pause": Click to freeze the image. "Stop": Click to stop playing back the recorded video. |
|  |  |  | / : Slow motion rewind/ forward. Slow reverse and forward (1x, 2x, 4x, 8x, 16x). |
|  |  |  | / : Speed up rewind/ forward. Fast reverse and forward (1x, 2x, 4x, 8x, 16x). |
|  |  |  | / : Goto previous/ next record. Click to select the previous/ next recorded video. |
|  |  |  | / : Step back/ forward. Click to play a recorded video step by step in the reverse/ forward direction. |
|  |  |  | The Split-Screen display function bar: This allows you to display the connected device in the multi-format screens of 1-way (1), 4-way (2x2), 9-way (3x3) and 16-way (4x4). |
|  | The display area. |  |  |
|  | Snapshot: | Provides the image capturing function. Click on the right key of the mouse on display area to show this icon, then click left to capture a jpeg file of the video. |  |
|  | Full Screen: | In the Live View area, if you want to change/delete the videos of cameras or use the snapshot/ full screen function, you can click the right mouse button again on the live view area, then click left to select. |  |

1.4.5 Config

Press the  button to enter the Config setting page.

NOTE: After finishing the settings of each page, please click Save to save the parameters.

(1) User

Click the  icon to enter the User setting page.

| User List: |  |
| --- | --- |
|  | Add User: 1.) Click to open the Add User window. 2.) Enter the information of the Username, Password, Confirm Password and Description. 3.) Select the User Group if necessary, 4.) Now press “Add”. |
|  | Modify User: 1.) Click to open the Add User window. 2.) Enter the information of the Username, Password, Confirm Password and Description. 3.) Select the User Group if necessary. 4.) Now press “Modify”. |
|  | Delete User: 1.) Select a user from the user list. 2.) Now press “Yes”. |
| User Information: |  |
| Provides the selected user’s information of Username, Description and Group. |  |

(2) User Group

Click the  to enter the User Group setting page.

| User Group List: |  |
| --- | --- |
|  | Add Group User: 1.) Click to open the Add Group User window. 2.) Enter the information of the Group Name and Description, then select the Authority. 3.) Set the member list from the available user list. 4.) Now press “Add”. |
|  | Modify User Group: 1.) Click to open the Modify User Group window. 2.) Enter the information of the Group Name and Description, then select the Authority. 3.) Set the member list from the available user list. 4.) Now press “Modify”. |
|  | Delete User Group: 1.) Select a group from the group list .2.) Now press “Yes”. |
| Group Information: |  |
| Provides the selected user’s information of Group Name, Description and Member. |  |

(3) Custom

Click the  icon to enter the Custom setting page.

| User Interface Settings: |  |
| --- | --- |
| (1) | Language: Select a language from the drop-down list. |
| (2) | Date Format: Select the date format from the drop-down list. |
| (3) | Font: Click to select the font. |
| Path Settings: |  |
| (1) | Folder for Snapshot Images: Click to select |
| (2) | Folder for Export AVI Clips: Click to select the path to save the AVI files. |
| Image Settings: |  |
| Select the aspect ratio: ( NOTE: The STRETCH status : Images are resized to full screen status. The aspect ratio status: When you resize images, they will retain their natural proportions. ) |  |
| (1) | AUTO: The JPEG compression mode comes with the STRETCH status. The MPEG 4 and the H264 compression modes have the aspect ratio status. |
| (2) | STRETCH: This status enables images in the MJPEG, MPEG4 and H264 modes to expand to the full screen. |
| (3) | FORCE TURN ON: This status enables images in the MJPEG, MPEG4 and H264 modes to remain in the aspect ratio status. |
| Sequence Settings: |  |
| Set the time duration (30, 60, 120, 180 or 300 seconds) from the drop-down list. |  |
| Save: Apply the changes. |  |
| Undo: Undo the latest changes |  |
| Restore Default: Recover to default setting. |  |

(4) Alarm Handler

Click the  icon to enter the Alarm Handler setting page.

Sound Settings:

(1) Tick mark the blank space to activate it. (2) To vary the sound file click to change it.

Record Settings:

(1) Tick mark the blank space to activate it. (2) Select the duration from the drop-down list. (3) Checkmark “PreAlarm” to activate the pre-alarm function.

SMTP Settings:

(1) Tick mark the blank space to activate it. Mail Server: Enter the mail server name. Recipients: Enter the recipient’s e-mail address. (2) Set the SMTP and the E-mail address to mail the user when the alarms occur.

Save: Apply the changes.

Undo: Undo the latest changes

Restore Default: Recover the default setting.

(5) Record Settings

Click the  icon to enter the Record setting page.

|  |  | Click to add a new record saving path. |
| --- | --- | --- |
|  |  | Click to delete a selected recording path. |
|  | Max. Usage: Set the maximum recording quota from the drop-down list. |  |
|  | Overwrite: Checkmark to activate the function. |  |
|  | Save: Apply the changes. |  |

(6) Logger

Click the  icon to enter the Logger listing page.

Example:

(7) Alarm List

Click the  icon to enter the Alarm listing page.

Example:

1.4.6 Tray

Press the  button to minimize the eneo GL-Manager monitor into the systray* of the Windows taskbar.

Right click the  button once to select the functions on the list: .

*: The Microsoft Windows systray is a portion of the Windows 95, Windows 98, Windows ME, Windows NT, Windows 2000, and Windows XP Operating Systems that help display running programs. The systray is located on the taskbar and is commonly in the bottom right hand corner of the screen next to the time display.